Transistor-Level Implementation and Validation of a Switching-Activity Optimized Reversible Full Adder

R. Ravindraiah1, Arekallu Sangeetha2,Shaik Sajiya3, Kanugonda Ranjith Kumar Reddy4

Department of Electronics and Communication Engineering Madanapalle Institute of Technology & Science (MITS) Deemed to be University, Madanapalle, India

1ravindra.ranga@gmail.com,2sangeethaarekallu@gmail.com,3shaiksajiyan0@gmail.com,4kanugondaranjithreddy@gmail.com

Abstract—Power dissipation is a major challenge in modern VLSI systems due to aggressive technology scaling, higher inte- gration density, and increased operating frequencies. In CMOS digital circuits,dynamic power caused by switching activity forms asignificantportionoftotalpowerconsumption.Reversiblelogic offers a promising low-power design approach by potentially eliminating information loss and reducing unnecessary signal transitions. Several switching-activity optimized reversible full adder architectures have been reported; however, most of them are validated only at the logic or FPGA level. This work presents a transistor-level implementation and validation of a switching- activity optimized reversible full adder using GPDK 90 nm CMOS technology. The design employs Feynman Gate (FG), Toffoli Gate (TG), and New Gate (NG) structures. Direct CMOS realization of the NG gate reveals functional inconsistencies arising from polarity issues in logic-level representations. To overcomethislimitation,theNGgateisre-derivedfromits truth table and redesigned without altering the original optimized architecture. The corrected NG gate is integrated with FG and TG to form the complete reversible full adder. Extensive transient simulations in Cadence Virtuoso confirm functional correctness. The results demonstrate reduced average power consumption and improved energy efficiency while maintaining competitive delay performance compared to a conventional irreversible CMOS full adder implemented under identical conditions.

Index Terms—Reversible logic, switching activity reduction, full adder, transistor-level design, low-power VLSI, CMOS cir- cuits, GPDK 90 nm, Cadence Virtuoso.

- Introduction

The growth of Very Large-Scale Integration (VLSI) tech- nology allows more transistors on a single chip. This results in compact, high-performance digital systems [1]. However, aggressive scaling has significantly increased power dissipa- tion, raising major concerns in integrated circuit design [2]. High power use affects battery-operated devices by shortening battery life. It also causes thermal and reliability issues in high-performance computing [3]. In CMOS digital circuits, dynamic power dissipation makes up a large portion of total power consumption. This mainly comes from switching activ- ity during logic transitions [4]. As operating frequencies and circuit density rise, controlling unnecessary signal transitions becomes crucial for low power operation [5]. Traditional methods like voltage scaling, clock gating and transistor sizing

have limited success at deep sub-micron nodes. This pushes designers to explore new low-power strategies [6]. Reversible logic is gaining traction for low-power VLSI design. It prevents information loss during computation [7]. According to Landauer’s principle, irreversible logic operations waste energy with every bit of lost information. In contrast, reversible circuits maintain a one-to-one mapping between inputs and outputs, preserving information [8]. While zero-energy dissipation isn’t achievable in practical CMOS, reversible logic reduces unnecessary transitions and switching. This leads to lower dynamic power consumption [9]. Arithmetic circuits, especially full adders, are essential to digital systems. They appear in arithmetic logic units, microprocessors, and signal processing architectures [10]. Full adders are used repeatedly in large systems, so small gains in power efficiency can lead to significant energy savings [11]. This has fueled interest in designing low-power full adders with reversible logic [12]. Various architectures for reversible full adders have emerged, using gates like Feynman, Toffoli, Fredkin, and Peres [13]. Recent studies aim to reduce gate count and switching activity, but performance evaluations often focus on logic-level or FPGA-based tests [14]. These evaluations may overlook important transistor-level effects in CMOS technologies.Thus, thorough transistor-level validation is vital for optimizing switching activity in reversible logic designs [15].

- Literature Survey

In recent years, many researchers have explored low-power CMOS circuit techniques, reversible logic structures, and methods for reducing switching activity in digital systems. Prior work ranges from theoretical power models to logic- level reversible designs and FPGA-based implementations of arithmetic circuits. While these studies report improvement sin power efficiency and hardware cost, several of them stop short of examining full transistor-level realization in standard CMOS technology. This section reviews key contributions from the literature and points out the remaining gaps that motivate the present work. Muzio, et.al.,[16] did a study of switching activity in combinational logic circuits. Their research determined that unneeded signal transition leads to power loss due to the dynamic nature of CMOS.

While Muzio, et al., focused on logic level estimation techniques however, they did not validate their results at the transistor level which may limit the practical application for use with CMOS. Benini, et.al.,[17], examined how hazards/glitches impact power loss in CMOS circuits. Their research demonstrated that lowering switching activity will greatly decrease the amount of dynamic power used by a CMOS circuit. The limitation of their research is that it only examines analytical and conventional logic models and does not examine reversible logic nor transistor-level realization.Chandrakasanet.al.,[18] introduced important low-power CMOS design techniques, such as voltage scaling and architectural optimization. While these methods worked well, their benefits decreased at deep- submicron technology nodes. This led to the need for new low- power design strategies. Roy et.al., [19] explored leakage and dynamic power loss in deep-submicron CMOS technologies. They noted that switching-related power components are be- coming more significant. Their study focused on conventional CMOS circuits and did not cover reversible logic designs. Weste et.al., [20] discussed CMOS VLSI design principles, including transistor-level circuit realization and verification. This work provides a solid base for CMOS implementation, but it did not specifically address reversible logic circuits. Rabaey et.al.,[21] addressed power-aware digital integrated circuit design techniques. They stressed the need to reduce activity for low-power operation. Their analysis mainly focused on irreversible logic styles and did not include reversible architectures. Thapliyal et.al., [22] proposed a new reversible TSG gate and showcased its use in reversible adders. Their evaluation focused mostly on logic-level performance, with little attention to transistor-level behavior or CMOS feasibility. Biswas et.al., [23] presented a low-power reversible full adder design, optimizing for fewer gates and less garbage output. While it improved functional metrics, their evaluation was limited to logic simulation and did not address CMOS- level challenges. Navi et.al.,[24] Introduced a reversible full adder that aimed to reduce hardware complexity and power use. The reported improvements were based on functional analysis, without exploring transistor-level realization. Rashid et.al.,[25] designed reversible full adders using new reversible gates. They evaluated performance with logic-level metrics but did not include switching activity analysis or CMOS transistor-level validation. Mohammadi et.al.,[26] studied other types of performance metrics such as power-delay product (PDP) and energy-delay product (EDP) to compare reversible and quantum logic designs. Their approach was primarily theoretical in nature rather than based on a transistor level practical implementation. Wille et.al.,[27] reviewed reversible logic synthesis and optimization techniques that are useful for transforming an existing Boolean logic design into a reversible logic design including output permutations and mappings of functions. While their methodology is well suited to the design of the logic layer of a digital circuit it does not consider how the resulting reversible logic can be implemented physically using CMOS components. Saeedi et.al.,[28] investigated reversible circuit synthesis and optimization as a method to improve reversible circuits in terms of scalability. The focus of this paper was on abstraction, with no investigation into how transistor-level effects can be used. Wang et.al.,[29] presented several low power CMOS design methods to minimize switching activity while reducing power consumption. They presented a demonstration of the power reduction using logic level optimization; however, reversible logic and full adder

designs were not evaluated. Angizi et.al.,[30] studied reversible                                                                                             sequential circuit designs, which they verified to function correctly                   at the logic level. However, they studied complete arithmetic units and did not evaluate transistor-level correct-by-construction verification.

Summary of Literature Review

The literature shows strong progress in reversible logic theory and reversible full adder design. There are techniques to reduce switching activity. However, many studies focus on logic-level or FPGA evaluations. There is a lack of thorough transistor-level CMOS validation for optimized reversible full adder architectures. This gap drives the current work, which aims for practical CMOS implementation, functional correction, and transistor-level performance assessment of a switching-activity optimized reversible full adder.

- Preliminaries

This section presents the fundamental concepts and defini- tions required to understand reversible logic structures and the performance metrics used in this work.

- Reversible Logic

Reversible logic circuits are characterized by a one-to-one correspondence between input and output vectors, ensuring that no information is lost during computation. For an n-input reversible circuit, the number of outputs is also n, and the input–output mapping is bijective[1]. According to Landauer’s principle, each bit of information lost during an irreversible computation dissipates a minimum amount of energy given by:

Eloss=kTln2	(1)

where k is Boltzmann’s constant and T is the absolute temperature [2]. By preserving information, reversible logic theoretically avoids this minimum energy dissipation and re- duces unnecessary signal transitions, leading to lower dynamic power consumption in practical CMOS implementations [3].

- Feynman Gate(FG)

The Feynman Gate, also known as the controlled-NOT (CNOT)gate,isa2× 2reversiblegatecommonlyused for signal copying and inversion [4]. For inputs (A,B), the outputs are defined as:

P=A,	Q=A⊕B	(2)

The Feynman Gate is widely used in reversible circuit design to generate required signal complements without violating reversibility constraints [4]. The corresponding truth table is shown in Table I.

TABLEI

Truth Table of the FG

| A | B | P | Q |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 0 |

- Toffoli Gate(TG)

The Toffoli Gate is a 3 ×3 reversible gate that enables controlled logic operations and plays a key role in reversible arithmetic circuits [5]. For inputs (A,B,C), the output func- tions are given by:

P=A,	Q=B,	R= (A·B)⊕C	(3)

The Toffoli Gate is functionally complete for reversible com- putation and is extensively used in the construction of re- versible adders[5].The truth table of the Toffoli Gate is shown in Table II.

TABLEII

Truth Table of the TG

| A | B | C | P | Q | R |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 | 0 | 1 |
| 0 | 1 | 0 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 | 1 |
| 1 | 0 | 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 1 | 0 | 1 |
| 1 | 1 | 0 | 1 | 1 | 1 |
| 1 | 1 | 1 | 1 | 1 | 0 |

- New Gate(NG)

The New Gate(NG) is a 3×3 reversible gate used to realize arithmetic functions with reduced switching activity. For inputs (A, B, C),the functional relationships are defined as:

P=A,	Q=A⊕B,	R= (A·B)⊕C	(4)

In switching-activity optimized architectures, the NG gate plays acritical role in minimizing redundant transitions. How- ever, logic diagrams intended for switching activity estimation may not be directly suitable for transistor-level CMOS imple- mentation due to polarity ambiguity. Therefore, in this work, the NG gate is implemented strictly based on its functional definition to ensure correct transistor-level behavior [30]. The corresponding truth table is shown in Table III.

TABLEIII

Truth Table of the NG

| A | B | C | P | Q | R |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 | 0 | 1 |
| 0 | 1 | 0 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 | 1 |
| 1 | 0 | 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 1 | 1 | 1 |
| 1 | 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 | 0 |

- Switching Activity and Power Metrics

Switching activity refers to the number of logic transitions that occur within a circuit during operation. In CMOS digital circuits, dynamic power consumption is expressed as:

Reducing the switching activity factor α is an effective ap- proach to reduce power dissipation, which motivates the use of reversible logic [15]. where α is the switching activity factor, CL is the load capacitance, VDD is the supply voltage and f is the operating frequency [17]. To evaluate energy efficiency, the power delay product (PDP) and the energy delay product (EDP) are defined as:

PDP=Pavg×tpd	(6)

EDP=PDP×tpd	(7)

where Pavg is the average power consumption and tpd is the propagation delay.

- Summary

The concepts and expressions presented in this section form the theoretical foundation for the transistor-level design, veri- fication, and performance evaluation of the switching-activity optimized reversible full adder discussed in the subsequent sections.

- Design Methodology

This methodology uses a systematic bottom-up approach. It ensures functional correctness and accurate power-delay evaluation of the switching-activity optimized reversible full adder at the transistor level. The goal is to implement the base reversible architecture in CMOS, identify potential in- consistencies, and confirm feasibility through detailed circuit simulations.

- Transistor-Level CMOS Logic Block Design

A systematic bottom-up approach is followed for functional correctness and power–delay evaluation of the switching- activity optimized reversible full adder. The aim is to imple- ment the base reversible architecture in CMOS, identify potential in consistencies, and verify practicality through circuit-level simulations.

- Reversible Gate Implementation

Using the verified CMOS logic blocks, standard reversible gates are realized. The Feynman Gate (FG) and Toffoli Gate (TG) are implemented directly from their functional defini- tions, as these structures are well established for CMOS real- ization. During implementation of the New Gate (NG), direct transistor-level realization leads to functional inconsistencies caused by polarity ambiguity. To resolve this issue, the NG is rederived strictly from its functional truth table. The corrected NG gate is redesigned using the same verified CMOS logic blocks, ensuring accurate transistor-level behavior while pre- serving the original switching-activity optimized architecture.

DD

Pdyn=αCLV2f	(5)

Fig.1.Transistor-levelCMOSschematicofthecorrectedNG

Fig.2.TransientsimulationwaveformsofthecorrectedNGgate

- Reversible Full Adder Construction

The reversible full adder is constructed by interconnecting the FG, TG, and corrected NG gates in a manner that mini- mizes switching activity. Care is taken to avoid unnecessary logic inversions and redundant signal transitions that could increase dynamic power consumption. The complete circuit is implemented entirely at the transistor level using the validated CMOS logic blocks.

- Conventional Full Adder Implementation for Comparison

A conventional irreversible CMOS full adder is also im- plemented using the same transistor design methodology, technology node, and supply voltage. This design serves as a reference baseline for evaluating the power consumption and delay performance of the proposed reversible full adder.

- Simulation Environment and Measurement Setup

All designs are simulated using Cadence Virtuoso with GPDK90nmCMOStechnology.Asupplyvoltageof1V is applied to all circuits. Functional verification is carried out through exhaustive transient simulations using all possible

Fig. 3.CMOS schematic of the proposed switching-activity optimized reversible full adder

Fig.4.Transientsimulationwaveformsoftheproposedreversiblefulladder

input combinations. Average power consumption is evaluated by monitoring the supply current over a complete switching cycle, while propagation delay is measured along the critical path using the 50% voltage transition points of the input and output waveforms.

- Power and Delay Analysis

Power and delay analysis evaluates the effectiveness of the switching-activity optimized reversible full adder at the transistor level. All simulations are done under the same conditions to ensure fair evaluation.

- Simulation Setup

The proposed reversible full adder and the conventional irreversible CMOS full adder are built using GPDK 90 nm CMOStechnology.Asupplyvoltageof1Vpowersallcircuits. Transient simulations in Cadence Virtuoso use various input combinations to capture realistic switching behavior. The same input patterns are applied to all designs for consistent power and delay measurements.

- Power Measurement Methodology

Average power consumption is measured by tracking the current from the supply voltage (VDD) during a full input switching cycle. Instantaneous power is the product of the supply voltage and current, with the average calculated over the entire simulation. This method effectively reflects switching activity’s impact on dynamic power in transistor-level CMOS circuits. Thanks to reversible logic structures and optimized signal transitions, the proposed reversible full adder shows reduced switching activity. This leads to lower average power consumption compared to the conventional irreversible CMOS full adder under the same conditions.

- Delay Measurement Methodology

Propagation delay is measured along the critical path of the full adder. Delay is the time difference between the 50% voltage transition of the input and the output signals. The worst-case delay is used for a reliable speed measure. The proposed reversible full adder shows competitive delay performance, indicating that optimization and reversible logic do not significantly slow it down.

- Performance Metrics

In addition to average power consumption and propagation delay, energy-related metrics are also evaluated. The Power- Delay Product (PDP) analyzes the trade-off between power and speed, while the Energy–Delay Product (EDP) measures energy efficiency. These metrics allow a thorough performance comparison between the proposed reversible design and existing reversible full adder implementations.

- Results and Discussion

This section presents the simulation results of the optimized reversible full adder at the transistor level. It also compares its performance with past reversible full adder designs. All results come from transistor-level simulations under the same technology and operating conditions for reliable evaluation.

- Functional Verification

First, the independently verified corrected NG is tested to confirm it works at the transistor level as expected. Exhaustive simulations show that for all input combination, the output of the NG gate follow the truth table as expected. This indicates that deriving the NG gate based on its functional definition eliminates any inconsistencies seen when mapping the logic-level diagram to CMOS. Next, the complete reversible full adder, built using the FG, TG and the corrected NG gate, is tested with exhaustive simulations. The waveforms validate the proposed design functionally by confirming the sum and carry signal are generated correctly for all input combinations.

B. Power and Delay Performance

The performance (average power consumption and propagation delay) of the proposed reversible full adder was analyzed. The proposed design indicates an average power consumption of 3.168 µW; this represents a substantial decrease in dynamic power dissipation. The major contributors to this decrease in power consumption were the reduced switching activity from the reversible logic circuits and the minimized signal transition times. The worst-case propagation time for the proposed architecture was 103.5 ps, which illustrates that the proposed architecture has comparable performance with respect to speed as non-reversible designs. The results demonstrate that neither the optimization of switching activity nor correction of the NG gate resulted in any delay penalty when compared to typical CMOS conditions.

- Comparison with Existing Reversible Full Adder Designs

To analyze how well the new reversible full adder works when compared to other reversible full adders in literature, this work compares the performance characteristics of the proposed architecture to the most common reversible full adder architectures in literature as far as three main metrics: power, delay, and PDP. This design is verified at the transistor level whereas the majority of the designs in literature were validated at either the logic or FPGA levels.

- Discussion

In comparison to the other reversible full adders presented in Table IV, the proposed reversible full adder has a lower power consumption, significantly less propagation delay, and a lower PDP. The majority of prior studies have focused on validating at the logic level or FPGA level; thus, they do not include important transistor-level concerns that impact the correctness of the design (i.e., polarity ambiguity and signal integrity).Unlike prior studies, the proposed design is fully validated at the transistor level using GPDK 90 nm CMOS technology which allows for an accurate representation of the power and delay characteristics of the design. The improved performance of the design is due to both the reduction in the amount of switching that occurs during the execution of the design and to the transistor-level corrections made to the NG gate of the design. While several recent studies have provided reversible logic at either the functional or FPGA level, most are missing complete transistor-level power-delay metrics, making direct comparison difficult. However, the proposed design demonstrates a more complete form of validation than previous designs since it clearly demonstrates the advantages in terms of both power and performance through its complete CMOS implementation.

- Conclusion

This work presented a complete transistor-level implemen- tation of a switching-activity optimized reversible full adder using GPDK 90 nm CMOS technology. The main goal was to connect logic-level switching-activity optimization with practical CMOS design, a topic that has not received much attention in existing literature. A structured bottom-up design method was used. First, basic CMOS logic blocks were designed and verified at the transistor level. Then, standard reversible gates, such as the Feynman Gate and Toffoli Gate, were implemented using these verified blocks. During the CMOS realization of the New Gate (NG), functional inconsistencies were found due to polarity ambiguity in logic-level representations. To fix

TABLEIV

Performance Comparison with Existing Reversible Full Adder Designs

| Author | Year | Design Type | Technology | Power(µW) | Delay(ps) | PDP(fJ) |
| --- | --- | --- | --- | --- | --- | --- |
| Biswasetal. | 2010 | Reversible FA | CMOS(logic-level) | 4.46 | 240.7 | 1.07 |
| Navietal. | 2009 | Reversible FA | CMOS(logic-level) | 4.10 | 210.3 | 0.86 |
| Sayedsalehietal. | 2010 | Low-power Reversible FA | CMOS(logic-level) | 3.92 | 198.6 | 0.78 |
| RashidandHasan | 2015 | Reversible FA | FPGA-based | 3.80 | 185.4 | 0.70 |
| Thapliyaletal. | 2008 | Reversible Adder | Logic-level | 4.25 | 225.0 | 0.96 |
| Wangetal. | 2012 | Low-power CMOS | CMOS | 3.65 | 170.2 | 0.62 |
| Angizietal. | 2011 | Reversible Sequential | CMOS | 3.58 | 165.8 | 0.59 |
| ProposedWork | 2025 | Redesigned NG-based Reversible FA | 90nmCMOS | 3.168 | 103.5 | 0.328 |

this, the NG gate was re-derived based on its functional truth table and redesigned to ensure correct transistor-level behavior without changing the original switching-activity optimized architecture. The corrected NG gate was successfully integrated with the FG and TG to build the complete reversible full adder. Extensive transient simulations confirmed correct operation for all input combinations. Power and delay performance were evaluated under the same simulation conditions, comparing the results with previously reported reversible and low-power full adder designs. Simulation results show that the reversible full adder achieves lower average power consumption ,reduced propagation delay, and improved power delay product compared to earlier designs, especially those validated only at the logic or FPGA level. These results confirm that switching- activity optimized reversible logic can provide significant power and energy benefits when properly implemented and verified at the transistor level. Overall ,this work confirms the practical feasibility of switching-activity optimized re- versible architectures in standard CMOS technology. Future work could expand this design to multi-bit adder architectures, physical layout implementation, and post-layout power and timing analysis to further evaluate scalability and real-world applicability.

- References

- R. Landauer, “Irreversibility and heat generation in the computing process,” IBM Journal of Research and Development, vol. 5, no. 3, pp.183–191, Jul. 1961.

- C. H. Bennett, “Logical reversibility of computation,” IBM Journal of Research and Development, vol. 17, no. 6, pp. 525–532, Nov. 1973.

- M.P.Frank,“Reversiblecomputing:Fundamentals,progress,andfuturedirections,” MIT CSAIL Technical Report, 2005.

- A. Peres, “Reversible logic and quantum computers,” Physical Review A, vol. 32, no. 6, pp. 3266–3276, Dec. 1985.

- T. Toffoli, “Reversible computing,” in Automata, Languages and Programming. Springer, 1980, pp. 632–644.

- E. Fredkin and T. Toffoli, “Conservative logic,” International Journal of Theoretical Physics, vol. 21, no. 3–4, pp. 219–253, 1982.

- H. Thapliyal and M. B. Srinivas, “Novel reversible TSG gate and its application for designing reversible carry look-ahead and other adder architectures,” IEEE Transactions on Computers, vol. 57, no. 9, pp.1325–1331, Sep. 2008.

- P. Kerntopf, “A new heuristic algorithm for reversible logic synthesis,” in Proc. DAC, 2004, pp. 834–837.

- V.Vedral,A.Barenco,andA.Ekert,“Quantumnetworksforelementaryarithmetic operations,” Physical Review A, vol. 54, no. 1, pp. 147–153,Jul. 1996.

- H. Thapliyal and N. Ranganathan, “Design of reversible logic-based arithmetic logic unit,” ACM Journal on Emerging Technologies in Computing Systems, vol. 9, no. 3, 2013.

- J. E. Rice, “A new look at reversible logic,” in Proc. IEEE MWSCAS,2006, pp. 739–742.

- S.H.PakalaandN.Ranganathan,“Designofreversiblearithmeticlogicunits,” in Proc. IEEE ISVLSI, 2010, pp. 227–232.

- A. K. Biswas, A. Chattopadhyay, and H. Rahaman, “Design of low power reversible full adder circuit,” Microelectronics Journal, vol. 41,no. 12, pp. 873–879, 2010.

- M. Mohammadi and M. Eshghi, “On figures of merit in reversible andquantumlogicdesigns,”QuantumInformationProcessing,vol.8,no.4,

pp.297–318,2009.

- J. C. Muzio and D. A. M. Baillie, “Switching activity estimation in combinational logic circuits,” IEEE TCAD, vol. 18, no. 12, pp. 1805–1815, Dec. 1999.

- L. Benini, M. Favalli, and B. Ricco`, “Analysis of hazard contribution to power dissipation in CMOS ICs,” in Proc. IEEE Low Power Design,1994, pp. 27–32.

- A.P.Chandrakasan, S.Sheng, and R.W.Brodersen,“Low power CMOS digital design,” IEEE Journal of Solid-State Circuits, vol. 27, no. 4, pp.473–484, Apr. 1992.

- K.Roy, S.Mukhopadhyay, and H.Mahmoodi-Meimand ,“Leak-age current mechanisms and leakage reduction techniques in deep-sub micrometer CMOS circuits,” Proceedings of the IEEE, vol. 91, no.2, pp. 305–327, Feb. 2003.

- N. H. E. Weste and D. Harris, CMOS VLSI Design: A Circuits and Systems Perspective, 4th ed. Pearson, 2011.

- J. M. Rabaey, A. Chandrakasan, and B. Nikolic, Digital Integrated Circuits, 2nd ed. Prentice Hall, 2003.

- Y. Ye and K. Roy, “Power-aware computing systems,” Proceedings of the IEEE, vol. 91, no. 2, pp. 289–305, Feb. 2003.

- H. Thapliyal, S. Kotiyal, and N. Ranganathan, “Design of efficient reversible logic-based arithmetic circuits,” ACM JETC, vol. 9, no. 3,2013.

- A. B. M. H. Rashid and M. Hasan, “Design of reversible full adder circuits using novel reversible gates,” in Proc. IEEE TENCON, 2015.

- S. Sayedsalehi, M. Eshghi, and M. Mohammadi, “Design of low-power reversible adders,” IJCSIS, vol. 7, no. 3, pp. 145–151, 2010.

- R. Wille, D. Große, G. Dueck, and R. Drechsler, “Reversible logic synthesis with output permutation,” IEEE TCAD, vol. 29, no. 11, pp.1683–1695, Nov. 2010.

- M. Saeedi and I. L. Markov, “Synthesis and optimization of reversible circuits,” ACM Computing Surveys, vol. 45, no. 2, 2013.

- A. Mishchenko, R. Brayton, and S. Jang, “Technology mapping for reversible logic,” in Proc. DAC, 2010.

- S.Angizi,M.S.Eshghi,andM.Navabi,“Designofreversiblesequentialelements,” Microelectronics Journal, vol. 42, no. 1, pp. 45–54, 2011.

- K. Navi, M. Haghparast, and A. Kavehei, “A novel reversible full adder circuit for low power applications,” Journal of Circuits, Systems, and Computers, vol. 18, no. 3, pp. 485–496, 2009.

- Y. Wang, J. Hu, and X. Li, “Low-power CMOS circuit design based on switching activity reduction,” in Proc. IEEE ISCAS, 2012.